Jay H. Troell

303 Hayden Road

Pleasanton, TX 78064

phone:  830/569-4946

December 3, 2015

Dr. Sanjeev Kalaswad

Texas Water Development Board

P.O. Box 13231

Austin, TX 78711-3231

Subject: HB 30 – Carrizo-Wilcox Aquifer

Dear Dr. Kalaswad:

Responding to your “Request for Public Input on Brackish Groundwater Production Zones” I have the following comments.

What is “significant impact”?  Leakage of fresh water into brackish groundwater due to pumping:  please note the attached chart illustrates how a relatively small change in Total Dissolved Solids produces a significant loss of fresh water.  For example a 10% decline from 1300 TDS in the produced BGW results from 14% fresh water (350TDS – typical Carrizo) being produced with the BGW i.e. leaking fresh water into the BGW well.  A 15% decline from 1300 TDS is caused by 20.5% fresh water leakage.  I submit that is “significant impact”.

The attached chart allows visualization of the effect of a decrease in TDS in produced brackish water due to “leakage” of fresh water into the BGW well.  The horizontal axis is the concentration of dissolved solids in the raw brackish water;  the vertical axis is the volume percent (%) of fresh water in the produced brackish water;  the parameters (1000, 1100, 1200 & 1300 ppm) represent the TDS of the produced brackish well water.  To use the chart: 1) determine the raw brackish water TDS, 2) measure the TDS in the produced water, 3)  using these values determine the volume percent (%) of 350 ppm “fresh” water in the produced BGW.

I have developed an EXCEL spreadsheet with the formulas necessary to determine the exact amount of fresh water leakage for any particular concentration of BGW and produced water.  Please advise if you would like the EXCEL spreadsheet file forwarded for your use.

Monitoring the TDS of a BGW well is necessary to ensure that a “significant impact” is caught before further damage occurs.  Anytime “significant leakage” is detected erosion and channeling of the sand matrix will increase the leakage of fresh water into the BGW production well.

SAWS sponsored a Black and Veach study that developed a model indicating over 800 ft. drawdown in a 50-well field producing BGW in the Wilcox aquifer in Wilson County.  The possibility of fresh water leakage due to non-continuous shale, faulting, non-conformity and ancient deep stream erosion between the fresh zone and BGW will be magnified by this cone of depression.

The Evergreen UWCD rules call for monitoring wells in close proximity to each BGW well.  The monitor wells should be located in the fresh water zone directly above the BGW.  This allows for early detection of fresh water drawdown i.e. “leakage”.

For a large BGW well-field additional monitor wells should be placed around the perimeter of the field in concentric rings to determine where and how far the fresh water drawdown extends.  Evergreen Rule 5.7, Large Scale Groundwater Pumping Projects, calls for monitoring wells along the property boundary.  If the Baseline Static Water Level falls 50 feet below the BSWL for three (3) consecutive months, the District may temporarily suspend the permit until the water level rises back to or above the BSWL.  If the level falls 75 feet below the BSWL the District may suspend the pumping for the duration of the permit period.

Implementation of HB30 particularly for the Carrizo-Wilcox should be based on the best available geologic studies;  I call your attention to the recent Hargis stratigraphic studies of Atascosa and Wilson Counties for non-conformity and faulting in the Carrizo-Wilcox aquifers.

Sincerely,

Jay Troell, P.E.

Atascosa Director – Evergreen Underground Water Conservation District

Enclosures:  Chart of “Carrizo Leakage into Brackish Groundwater”