TWDB WAM Automation Program (ResFY_RWP)– User’s Menu

Yujuin Yang

TWDB, Yujuin.Yang@twdb.texas.gov, (512)936-2385

Functions

This automation program operates WAM (TCEQ’s SIM and TAB) to compute reservoir firm yields.  It has following functions:

1. compute firm yield for a single reservoir,

2. compute firm yield for a reservoir system,

3. compute firm yields for multiple reservoirs in order,

4. automatically identify water rights associated with reservoirs,

5. handle water right backup relationship,

6. add new water right to examine reservoir potential additional firm yield,

7. in addition to the firm yield, provide firm diversion of every water right in the firm yield,

8. option to output reservoir table or hydrograph when diversions at firm yield level.

- How the Automation Program Works

The automation program executes WAM (Sim and Table) simulates the process of estimating reservoir firm yield: it first identifies all water rights associated with the reservoir (system), then it tests each water right in seniority order to see if permitted diversion is firm (no shortage). If supply shortage occurs, it reduces the diversion and re-run WAM. This trial-and-error is repeated until a maximal diversion is found without causing shortage. At this point, it will go to the next most senior water right and on until firm diversions have been found for all water rights. The firm diversions are summed to obtain the firm yield. When a senior right is been tested, all rights junior to it will assume a zero diversion. When a firm yield for a particular reservoir is being assessed, all other reservoirs in the basin with divert at their permits.

ResFY_RWP.py is the controlling automation program, which calls (executes) SIM.exe and TAB.exe alternatively and repeatedly, according to the control parameters set in the input.dat and parameters_rwp.dat files.

For given reservoir / system, find all associated water rights (including backup rights)

Sort water rights based on seniority

Find the most senior right that causes supply shortage (“trouble maker”)

Find firm diversions for all rights through a trial-and-error

Sum firm diversions to obtain firm yield

Find firm diversions for all rights through a trial-and-error process – steps 4 to 6

- Automation Program files and input files

The automation run involves 3 executables (ResFY_RWP.exe, SIM.exe,  Tab.exe), and 2 parameter files (input.dat,  parameter_rwp.dat), in addition to WAM input data files.

- Parameter files

  - parameter.dat includes 5 run control parameters. See example below:

Trinity		# river basin name

C:/WAMStudy/,	# root path where all computation occurs

extra_fy=no,	#add a new, junior reservoir water right? yes or no

extra_div=0,        	#annual diversion in acre-feet.

Res_output=no,	#Output reservoir table? yes or no

plot_display=no,	#display reservoir storage plot on screen while program running? yes or no.

All ‘yes’ and ‘no’ are in lower case.

3.2 input.dat includes the root name of WAM input files and the list of reservoir IDs in WAM’s .dat file. See example below. Note the first one is a reservoir system with 6 member reservoirs. If reservoir ID is less than 6 character-long, make it 6 character-long and modify in WAM .dat file correspondently.

C3

MARBLETRAVISBUCHANLKAUSTROYINKLAKLBJ

THOMAS

SPENCE

CHAMPI

…

BALLIN

ELMCRK

BRADYC

LCLYDE

HORDSC

- Output files

The automation runs will produce final output files as well as intermediate output files.

Final output files include:

A summary file that contains firm yields for all reservoirs listed in input.dat;

A record file with firm diversions and records of iteration details for each water right of all reservoirs, and

reservoir storage tables and plots correspondent to the diversions at firm yield level (if Res_output=yes).

The Intermediate output files include:

mid.xxx files which are SIM’s input and output file,

tab.xxx which are Table’s input and output files,

simu.txt and table.txt,

error_message.out records errors when Table run is incomplete,

runwam.bat

- Directory structure

Users must prepare directory folders before running the program. It should have root folder, a basin folder, and two sub-folders under basin folder. For example:

- Root folder: C:/WAMStudy/

- Basin folder: C:/WAMStudy/Trinity/

- Sub-folders under basin folder: C:/WAMStudy/Trinity/Data and C:/WAMStudy/Trinity/Result

In Basin folder, stores

- Automation program (ResFY_RWP.exe),

- WAM executables (SIM.exe and Tab.exe),

- input.dat,

- parameter_rwp.dat;

Automation program will generate an error message file (error_message.out) and other intermedia output files in this folder;

In /Data folder, stores all WAM input data files (.dat, .eva, .dis, .inf, .flo, .fad, .his, etc.)

In /Result folder, stores all output files that the automation program will generate.

- Run ResFY_RWP

To run this automation program, follow the steps below:

(1). Prepare the folders and the executable and data files as instructed in <5. Directory structure>;

(2). Re-name input_(basin name).dat to input.dat and check if your reservoir(s) are in the file; if not, add them. Make sure reservoirs in a system are entered in one line together. Remove the ones you do not wish to estimate the firm yield for;

(3). Re-name sample_parameters_rwp.dat to parameters_rwp.dat and check the river basin name and path to your root folder in it to make sure they are correct;

(4). If input.dat contains multiple reservoir lines, set Res_output and plot_display to “no”;

(5). Double click ResFY_RWP.exe to execute it.

(6). After computation is completed, check the output file with firm diversions for each water right to exclude water rights that you do not believe should be included in firm yield (for example, rights to simulate subordination agreements, etc.).

- Tips

- If a reservoir ID is less than 6-character long, fill the empty spaces to make it 6-character long in input.dat and WAM’s .dat file. For example, make KEMP KKKEMP.

- If in WAM’s .dat file a PX card is set to one (PX=1. For example in Brazos and Trinity WAM), deactivate the DUAL option by setting -1 for DUALD in field 16 in the JO card.

- In 2018 Brazos WAM, deactivate DSSRU (set to blank) if .RUF file is not provided.

- Use 2018 version WAM (SIM and TAB).